2026 Air Quality Annual Status Report (ASR)

In fulfilment of Part IV of the Environment Act 1995 Local Air Quality Management, as amended by the Environment Act 2021

Date: 30 June, 2026

| Information | North Tyneside Council Details |
| --- | --- |
| Local Authority Officer | Frances McClen Claire Wilson |
| Department | Environmental Health |
| Address | Quadrant East, The Silverlink North, North Tyneside, NE27 0BY |
| Telephone | 0191 643 6100 |
| E-mail | Environmental.health@northtyneside.gov.uk |
| Report Reference Number | Cw/Air.0026 |
| Date | 30 June 2026 |

## Local Responsibilities and Commitment

This ASR was prepared by the Environmental Health Department of North Tyneside Council with the support and agreement of the following officers and departments:

- Integrated Transport Team

- Development Control

- Environmental Sustainability Officer

- Public Health

This ASR has been approved by:

Joanne Lee, Head of Public Protection, Community Safety and Emergency Planning

This ASR has been signed off by a Director of Public Health.

If you have any comments on this ASR please send them to Frances McClen or Claire Wilson at:

Environmental Health, Public Protection

Quadrant East, The Silverlink North

North Tyneside, NE27 0BY

0191 643 6100

Environmental.health@northtyneside.gov.uk

# Executive Summary: Air Quality in Our Area

## Air Quality in North Tyneside

Breathing in polluted air affects our health and costs the NHS and our society billions of pounds each year. Air pollution is recognised as a contributing factor in the onset of heart disease and cancer and can cause a range of health impacts, including effects on lung function, exacerbation of asthma, increases in hospital admissions and mortality.

Air pollution particularly affects the most vulnerable in society, children, the elderly, and those with existing heart and lung conditions. Low-income communities are also disproportionately impacted by poor air quality, exacerbating health and social inequalities.

Table ES 1 provides a brief explanation of the key pollutants relevant to Local Air Quality Management and the kind of activities they might arise from.

Table ES 1 - Description of Key Pollutants

| Pollutant | Description |
| --- | --- |
| Nitrogen Dioxide (NO2) | Nitrogen dioxide is a gas which is generally emitted from high-temperature combustion processes such as road transport or energy generation. |
| Sulphur Dioxide (SO2) | Sulphur dioxide (SO2) is a corrosive gas which is predominantly produced from the combustion of coal or crude oil. |
| Particulate Matter (PM10 and PM2.5) | Particulate matter is everything in the air that is not a gas. Particles can come from natural sources such as pollen, as well as human made sources such as smoke from fires, emissions from industry and dust from tyres and brakes. PM10 refers to particles under 10 micrometres. Fine particulate matter or PM2.5 are particles under 2.5 micrometres. |

North Tyneside is located to the east of the regional capital of Newcastle upon Tyne. It is one of the five metropolitan districts that makes up the Tyne and Wear conurbation. The Borough is made up of 20 wards and has a residential population of around 212,000 with approximately 99,000 households and covers an area of 84km2. The Borough stretches from the eastern boundary of Newcastle upon Tyne to the North Sea and from the southern boundary of Northumberland to the River Tyne.

The northern fringe of the Borough is open countryside with the main urban areas, including the towns of Wallsend, North Shields, Tynemouth, and Whitley Bay located along the river and coastline. Additionally, there are three large settlements to the west of the Borough; Longbenton, Forest Hall and Killingworth and to the north of the Borough are the villages; Wideopen, Burradon, Annitsford, and Backworth.

The River Tyne is a commercial river with ship repair, offshore fabrication, fishing and port related industries. The riverside urban area has undergone major regeneration which has resulted in some diversification from ship building to offshore related construction work. Business parks were created along the main transport routes of the A19 and the coast road A1058 including Cobalt Business Park, Balliol Business Park and retail outlets including the Silverlink and Royal Quays. Residential areas have been developed on former industrial land adjacent to the heavy engineering yard Smulders in Wallsend, on the former Hayhole gas works at Wallsend and at The Limes, Great Lime Road, Palmersville. Residential developments have also been created on greenfield land adjacent to the A19 at Backworth and Scaffold Hill. New residential developments have been approved and are under construction on an area of land known as the Murton Gap, between Whitley Bay and New York, and on Killingworth Moor to the north of the Borough.

North Tyneside Council has 61 smoke control areas that cover the majority of North Tyneside. Smoke control areas make it an offence to create smoke from a chimney except during lighting up when kindling can be used. Further smoke legislation has been introduced restricting retail shops to sell only smokeless solid fuel, and for all wood sold in volumes of less than 2 cubic metres required to be certified as ‘Ready to Burn’. The Environmental Protection team will investigate and enforce compliance with the smoke control legislation to ensure no un-authorised fuels and/or appliances are being used within the smoke control areas. The use of solid fuel or wood burning stoves as a source of heating for domestic properties has increased over the last few years which could increase localised particulate emissions in residential areas. A resident can install a wood burner or multi fuel stove provided it is on the DEFRA list of exempted appliances and is installed by a competent person.

Smoke control enforcement remains a priority for the Authority. Three indicative air samplers are installed within residential areas to measure fine particulate levels in areas where there has been an increase in the use of wood burning stoves.

The main pollutants monitored within the Borough of North Tyneside arise from traffic emissions and are nitrogen dioxide (NO2) and fine particulates. Fine particulates are particulates of less than 2.5 microns in diameter (PM2.5) or 10 microns in diameter (PM10) and are breathed into our lungs.

Nitrogen Dioxide monitoring is carried out via indicative passive monitoring at roadside locations throughout the Borough. Monitoring shows NO2 concentration levels have shown a slight increase in 2025 at a number of the monitoring locations when compared with the period 2021 to 2024. All monitoring locations show North Tyneside continues to achieve good air quality. The nitrogen dioxide annual mean concentrations are well below the UK air quality objectives, including at the automatic monitoring station located at NTC01 on the Coast Road A1058. There were no new passive indicative NO2 sites introduced for the 2025 monitoring year.

North Tyneside Council has invested in five indicative air samplers to supplement monitoring for NO2 and particulates PM10 and PM2.5.  Two indicative air samplers are located about Holystone Way, A191 and three are located in residential areas of Wallsend, Tynemouth and Whitley Bay.  All indicative air samplers recorded concentrations well below the UK annual mean objective of 40 µg/m³ for NO2 and PM10, indicating good air quality in relation to nitrogen dioxide and particulates.  The air sampler WL located in Whitley Lodge, Whitley Bay, had a power failure during April 2025 and there was a delay in its reinstallation.  This meant that only 4 months data was recorded, so the monthly means for the 4 months are given in Tables C.6, C.7 and C.8 in Appendix C, but the annual mean is not provided. Air samplers HB1 and HB2 located on Holystone Way, which is a busy dual carriageway, showed similar levels for NO2, whereas the air samplers located in Richardson Street, Wallsend and Mariners Lane, Tynemouth, showed higher NO2 levels than HB1 and HB2, but all were well below the annual mean objective level of 40 µg/m³. HS1 had a slight increase in NO2 concentrations when compared with 2024 but a slight decrease in PM10 and PM2.5, whereas HS2 monitoring location the NO2 levels recorded were similar to 2024, whereas PM10 and PM2.5 concentrations increased marginally in 2025.  The PM2.5 and PM10 levels at all four monitoring locations are low, suggesting good air quality with respect to fine particulate matter.

The Authority has not declared any Air Quality Management Areas (AQMAs) within the Borough and the latest monitoring data has shown no requirement to proceed to declare any AQMAs.

Four planning applications were assessed in relation to air quality impacts. Application 24/01678/FULM involved the replacement with new plant and back up diesel generators for a database centre. The air quality assessment considered negligible construction impacts provided a construction management plan was implemented, and advised that air

quality modelling for the diesel generators to ensure compliance with air quality standards. The development will require an environmental permit which will regulate emissions. Application 25/00974/FUL for 8 residential flats found no significant impacts during construction (with mitigation) or operation due to low traffic generation. Similarly, planning application 25/01213/FULM for 82 homes concluded that impacts from both construction and traffic emissions would not be significant and pollutant levels would remain below objectives. In contrast, Application 25/01140/FULM for a mixed-use development remains under consideration, as operational air quality impacts have yet to be assessed due to missing traffic data, although construction impacts are expected to be negligible with appropriate controls. Further information on the planning applications is provided in Appendix C. There was one new industrial process granted an environmental permit in 2025, with further details on the process provided in Appendix C.

North Tyneside Council will continue to monitor nitrogen dioxide using a network of passive diffusion tubes, alongside real-time monitoring of nitrogen dioxide and particulate matter. Continuous measurements are undertaken at the automatic monitoring station on the Coast Road (A1058), supported by five indicative samplers located across four sites within the borough.

## Actions to Improve Air Quality

Whilst air quality has improved significantly in recent decades, there are some areas where local action is needed to protect people and the environment from the effects of air pollution.

North Tyneside Council has developed an Air Quality Strategy which sets out a programme of future actions aimed at reducing air pollution and improving public health outcomes. The Strategy adopts a multi-disciplinary approach, bringing together expertise from spatial and transport planning, environmental and public health services, as well as engaging local political leaders, community representatives, and residents. It establishes eight overarching aims supported by 31 targeted actions designed to deliver measurable improvements in air quality across the borough.

The key aims of the Strategy include maintaining and enhancing air quality and public health; reducing emissions associated with transport; integrating air quality considerations within planning, development, and land use policies; promoting improved health outcomes linked to better air quality; addressing emissions from fixed and industrial sources; encouraging public awareness and informed decision-making; supporting the delivery of the Carbon Net Zero Action Plan; and ensuring effective partnership working across sectors.

In line with the Air Quality Strategy, North Tyneside Council has continued to promote sustainable transport as an alternative to private vehicle use. A key achievement was the opening of the Seafront Sustainable Route in August 2025, delivering over 6 km of improved footways, 4.5 km of new cycleways, and 20 crossings along the coast between Tynemouth and Whitley Bay. The Authority also supported active travel to schools through pedestrian and Bikeability training, reaching over 16,000 children during the 2024/25 academic year.

Work has progressed in partnership with bus operators and Nexus to enhance public transport, including development of highway improvements funded through the Bus Service Improvement Plan, with delivery planned from 2026/27. Additional measures have included cycling and pedestrian infrastructure upgrades, improvements to public transport access, transition of bus fleets to electric vehicles, and exploration of an e-bike hire scheme.  In September 2025, £10.9m of funding was provided by the North East Combined Authority to make sure that the rail station on the Northumberland Line rail route was completed as planned.  The money delivers a brand-new station at Northumberland Park, connecting people in North Tyneside directly to the national rail network for this first time in decades as well as creating a seamless link with Metro and local bus services.

The Authority continues to embed air quality considerations within the planning process, with Environmental Health involved in consultations and pre-application discussions, and air quality assessments required for major developments. Progress is also being made on the new Local Plan, which includes strengthened policies on pollution control, climate change, and air quality. The draft plan places greater emphasis on mitigation measures, active travel integration, and green infrastructure such as tree planting to help reduce pollution and improve environmental quality.

Environmental Health continues to investigate smoke nuisance and breaches of the smoke control orders.  North Tyneside Council intends to introduce a smoke enforcement policy, which was anticipated to be actioned during 2025, but is now proposed to be progressed during 2026/27.  This will provide for an effective means to penalise offenders of smoke control offences using financial penalties and ensure action 1.4 of the Air Quality Strategy is achieved.

Three of the indicative continuous air samplers were installed in the residential areas of Whitley Bay, Tynemouth and Wallsend I 2025.  These are located in residential areas to   consider impact of wood burning smoke emissions and to support smoke enforcement and air quality awareness monitoring for particulates and NO2.  The air sampling is useful as a guide to provide an indication of air quality and pollutant levels in these areas.

## Conclusions and Priorities

North Tyneside Council are pleased to report that no exceedances of the Air Quality Objectives were identified during the year 2025. The Authority does not currently have any AQMA’s. The monitoring results for 2025 conclude that there continues to be no requirement to declare any AQMAs for any pollutants.

A review of the monitoring data over the last five years indicates a slight increase in NO2  and particulate levels in 2025 when compared with the period 2021-2024, but that all monitoring locations show levels well below the National Air Quality Objectives.

Our main priorities for 2026 are as follows:

North Tyneside Council will continue to focus on the aims and objectives of the Air Quality Strategy that highlights the Authority’s commitment to a proactive approach to improving air quality.

- North Tyneside Council remains committed to monitoring and review of the air quality in the borough gathered from the automatic and passive NO2 and particulate monitors.

- North Tyneside Council will further enhance the local air quality evidence base by reviewing monitoring locations and where necessary expand the monitoring coverage to consider changes via developments on pollution patterns and population exposure.

- North Tyneside Council will identify and prioritise pollution hotspots, using monitoring results to target interventions and resources in areas with the greatest risk to public health.

- North Tyneside Council will support compliance and enforcement, ensuring relevant regulations include smoke control and environmental permitting processes are effectively implemented to minimise emissions.

## How to get Involved

North Tyneside Council continues to actively engage with residents, stakeholders, and partner organisations to raise awareness of air quality issues and encourage participation in initiatives aimed at reducing pollution. This ongoing engagement supports a collaborative approach, ensuring that the needs and views of the community are reflected in local policy and decision-making. Air quality considerations are embedded within the Authority’s strategic planning framework, with the Air Quality Strategy informing  development management processes. This is further reinforced through its integration within the Local Plan, ensuring that new developments take full account of air quality impacts and incorporate appropriate mitigation measures from the outset.

The Authority is committed to maintaining transparency and accessibility of information relating to air quality. A range of resources, including current data, guidance on pollution control, and historical reports, are made publicly available via following weblink North Tyneside Air Quality. This platform ensures that residents and interested parties can easily access reliable and up-to-date information, supporting greater understanding of local air quality conditions and trends.

In addition, North Tyneside Council continues to promote sustainable travel choices as a key measure to improve both environmental quality and public health. Residents are encouraged to consider alternatives to car use, particularly for short journeys, with walking and cycling providing clear benefits in terms of physical health, mental wellbeing, and reduced emissions. To support this, the Authority has developed and continues to expand a network of safe and accessible cycle routes across the borough. These routes are actively promoted through the Authority’s website and other communication channels, helping to increase awareness and uptake of active travel options.

A Cycling strategy and Local Transport Strategy have been adopted by the Authority. Further information about these strategies can be obtained from the following website:

NTC Cycling Strategy, Transport strategy

Information on North Tyneside Council Climate Change and Sustainability Policies are available using the following link:

Climate Change Policies

Detailed information is also available on the government stance on air quality. Further information on Local Air Quality Management is available at the following web address:

https://uk-air.defra.gov.uk/

Table of Contents

Local Responsibilities and Commitment	i

Executive Summary: Air Quality in Our Area	i

Air Quality in North Tyneside	i

Actions to Improve Air Quality	vi

Conclusions and Priorities	ix

How to get Involved	x

1	Local Air Quality Management	1

2	Actions to Improve Air Quality	2

2.1	Air Quality Management Areas	2

2.2	Progress and Impact of Measures to address Air Quality in North Tyneside Council	6

2.3	PM2.5 – Local Authority Approach to Reducing Emissions and/or Concentrations	12

3	Air Quality Monitoring Data and Comparison with Air Quality Objectives and National Compliance	19

3.1	Summary of Monitoring Undertaken	19

3.1.1	Automatic Monitoring Sites	19

3.1.2	Non-Automatic Monitoring Sites	20

3.2	Individual Pollutants	20

3.2.1	Nitrogen Dioxide (NO2)	20

3.2.2	Particulate Matter (PM10)	22

3.2.3	Particulate Matter (PM2.5)	23

Appendix A: Monitoring Results	24

Appendix B: Full Monthly Diffusion Tube Results for 2025	40

Appendix C: Supporting Technical Information / Air Quality Monitoring Data QA/QC	42

New or Changed Sources Identified Within North Tyneside Council During 2025	42

Additional Air Quality Works Undertaken by North Tyneside Council During 2025	45

QA/QC of Diffusion Tube Monitoring	45

Diffusion Tube Annualisation	46

Diffusion Tube Bias Adjustment Factors	46

NO2 Fall-off with Distance from the Road	48

QA/QC of Automatic Monitoring	49

PM10 and PM2.5 Monitoring Adjustment	50

Automatic Monitoring Annualisation	50

NO2 Fall-off with Distance from the Road	50

Summary of Low Cost Air Quality Sensor Monitoring	51

Appendix D: Map(s) of Monitoring Locations and AQMAs	55

Appendix E: Summary of Air Quality Objectives in England	57

Glossary of Terms	58

References	59

Figures

Figure A.1 – Trends in Annual Mean NO2 Concentrations	34

Figure A.2 – Trends in Annual Mean PM10 Concentrations at Automatic Monitoring Location NTC01	37

Figure C.1 – National Bias Adjustment	48

Figure D.1 – Map of Automatic and Non-Automatic Monitoring Site	55

Figure D.2 – Map of Indicative Air Sampling Monitoring Locations	56

Tables

Table 2.1 – Progress on Measures to Improve Air Quality	10

Table A.1 – Details of Automatic Monitoring Sites	24

Table A.2 – Details of Non-Automatic Monitoring Sites	25

Table A.3 – Annual Mean NO2 Monitoring Results: Automatic Monitoring (µg/m3)	29

Table A.4 – Annual Mean NO2 Monitoring Results: Non-Automatic Monitoring (µg/m3)	30

Table A.5 – 1-Hour Mean NO2 Monitoring Results, Number of 1-Hour Means > 200µg/m3	35

Table A.6 – Annual Mean PM10 Monitoring Results (µg/m3)	36

Table A.7 – 24-Hour Mean PM10 Monitoring Results, Number of PM10 24-Hour Means > 50µg/m3	38

Table A.8 – Annual Mean PM2.5 Monitoring Results (µg/m3)	39

Table B.1 – NO2 2025 Diffusion Tube Results (µg/m3)	40

Table C.1 – New Industrial Sources	44

Table C.2 – Annualisation Summary	46

Table C.3 – Laboratory Summary Performance for AIR NO2 PT Rounds	47

Table C.4 – Bias Adjustment Factor	48

Table C.5 – Details of Low Cost Sensor Sites	51

Table C.6 – Annual and Mean Indicative Air quality Sensor Monitoring Results – NO2	51

Table C.7 – Annual and Mean Indicative Air quality Sensor Monitoring Results – PM10	52

Table C.8 – Annual and Mean Indicative Air quality Sensor Monitoring Results – PM2.5	54

Table E.1 – Air Quality Objectives in England	57

# Local Air Quality Management

This report provides an overview of air quality in North Tyneside Council during 2025. It fulfils the requirements of Local Air Quality Management (LAQM) as set out in Part IV of the Environment Act (1995), as amended by the Environment Act (2021), and the relevant Policy and Technical Guidance documents.

The LAQM process places an obligation on all local authorities to regularly review and assess air quality in their areas, and to determine whether or not the air quality objectives are likely to be achieved. Where an exceedance is considered likely the local authority must declare an Air Quality Management Area (AQMA) and prepare an Air Quality Action Plan (AQAP) setting out the measures it intends to put in place in order to achieve and maintain the objectives and the dates by which each measure will be carried out. This Annual Status Report (ASR) is an annual requirement showing the strategies employed by North Tyneside Council to improve air quality and any progress that has been made.

The statutory air quality objectives applicable to LAQM in England are presented in Table E.1.

# Actions to Improve Air Quality

Air Quality Management Areas

Air Quality Management Areas (AQMAs) are declared when there is an exceedance or likely exceedance of an air quality objective. After declaration, the authority should prepare an Air Quality Action Plan (AQAP) within 18 months. The AQAP should specify how air quality targets will be achieved and maintained, and provide dates by which measures will be carried out.

North Tyneside Council currently does not have any declared AQMAs. A local Air Quality Strategy is in place to prevent and reduce polluting activities. The Local Air Quality Strategy is available on the Authority’s website.

The aims and objectives of the air quality strategy are reviewed regularly through an air quality steering group, meeting twice yearly to discuss progress. This group is represented by Highways, Planning, Sustainability, Public Health and Environmental Health to discuss progress on the aims and actions of the strategy.

Air quality information is made available on the Authority’s website.  This information is reviewed annually and updated.

Aim 1.4 and 1.5 of the Strategy focuses on smoke enforcement. Three real time indicative air samplers monitoring for NO2 and particulates are located in residential areas where concerns over wood burning appliances and associated emissions had been raised to assess impact of localised emissions. The residential monitoring locations are in Tynemouth, Whitley Bay and Wallsend.  Tables C.6, C.7 and C.8 in Appendix C detail the monthly and annual means for NO2 and Particulates. The Whitley Lodge (WL) monitor experienced a power failure in April 2025, resulting in only four months of data so no annual mean can be derived. Richardson Street (RS) and Mariners Lane (ML), both had particulate levels well below the annual mean objective for particulates and NO2. Overall, PM₂.₅ and PM₁₀ levels at both sites with a full 12 months of data remained low, indicating good air quality and no evidence of increased particulate emissions from wood-burning stoves. Comparisons between the residential RS and ML monitoring sites and the road monitoring sites about Holystone bypass road A191, HS1 and HS2 sites, show similar particulate levels, suggesting that wood-burning stove use is not having a significant localised impact on air quality.

A smoke enforcement policy was to be developed in 2025-26, however, this has been delayed and is now anticipated to be implemented by 2027.

As part of the Public Sector Decarbonisation Scheme - The Authority has completed the replacement of end-of-life gas heating systems in two buildings with air source heat pumps and other energy efficiency measures. A solar photovoltaic (PV) array has been installed at Tynemouth Pool to utilise clean energy for the site. £2.9m has also been secured to decarbonise the Council headquarters and so reduce air emissions.

In partnership with the North East Community Forest, over 9 hectares of new trees were planted in the Borough which will absorb 90 to 270 tonnes of CO2 per year improving air quality.

The Authority secured £4.28m from the Government's Warm Homes Local Grant to improve the energy efficiency of 300 low-income homes over the next 3 years.

Following a public engagement event, the Authority published an Adaptation Action Plan for the borough. Approved by Cabinet in March, the plan builds on 2024's DEFRA pilot to widen the scope of climate resilience across North Tyneside.

In accordance with aim 2.1 of the strategy, the Authority has continued to support the greater use of more sustainable modes of transport in preference to motorised journeys. August 2025 saw the formal opening of the Sea Front Sustainable Route, which provides a two-way safe space for cycling suitable for families, alongside improved walking and wheeling provision, along North Tyneside’s coast between Tynemouth and Whitley Bay. The £11m scheme, which was fully funded by the Government’s Active Travel Fund and voluntary sector organisation Sustrans, included over 6km of resurfaced footways, over 4.5km of new cycleway and 20 new crossings.

Aim 2.2 of the strategy includes supporting active and sustainable travel to schools. The Authority continues to deliver its child pedestrian training and national ‘Bikeability’ cycling training. The training supports children to walk and cycle safely particularly for travel to and from school. In the 2024/25 academic year, child pedestrian training was delivered to 13,920 children in the borough and 2,116 children received Bikeability training.

Aim 2.3 of the strategy focuses on engaging with bus operators to progress the objectives of the North East Bus Service Improvement Plan (BSIP). The Authority has continued to work in partnership with local bus operators and Nexus. Design and business case work has been carried out in 2025/26 on three schemes to support the operation of junctions on the highway network which are used by local bus services. The schemes, such as junction remodelling and introducing traffic signals, have secured BSIP capital funding from the North East Combined Authority, with scheme delivery to commence in 2026/27.

Other measures implemented included cycling infrastructure improvements, public transport access improvements, bus fleet upgrades to electric vehicles, footpath upgrades and an initiative to investigate the potential for borough wide ebike hire scheme and leasing options.

Aim 3.1 of the Strategy provides for pre-application discussions.  Environmental health continues to be consulted on planning applications. With large developments requiring air quality assessments. Pre-application discussions continue to be encouraged.

Aim 3.4 focuses on the Local Plan. As of January 2026, the Local Authority is progressing the preparation and adoption of a new Local Plan. Environmental Health is working closely with the Planning team to help identify and shape policy changes relating to air quality.

The initial draft Local Plan, published in January 2026, sets out a series of objectives for the borough, including objectives that specifically address air quality, climate change and pollution. Policy DM14.1 (Pollution) has been strengthened to align with national standards and now requires robust supporting evidence, the application of appropriate mitigation measures, and monitoring for major developments.

There is also an increased emphasis on incorporating active travel into the design and layout of development sites to reduce transport-related emissions. In addition, the policy gives greater priority to landscaping and green infrastructure, including tree planting, to help mitigate the impacts of air pollution.

Progress and Impact of Measures to address Air Quality in North Tyneside Council

Defra’s appraisal of last year’s ASR concluded that:

- The 2025 report was well structured and acknowledged the robust AQ/QC procedures.

- The co-location of the diffusion tube monitoring with the automatic monitor located on the Coast Road NTC01 referred to in the 2025 ASR report is to be implemented for the 2027 monitoring programme for validating nitrogen dioxide results.

- Recognition was given to the Authority’s commitment to air quality monitoring and measures to improve air quality, for both NO2 and Particulates including PM2.5 despite the low pollutant concentration levels.

- It was recommended that layout and design of tables is reviewed to ensure they are more presentative, ensuring that all data required is included.  This has been taken on board.

- It was also suggested that for future ASR reports that a table is included for the AIR PT data, this again has been noted.

The comments of appraisal have been noted and actioned, except for use of a co-location of the diffusion tube monitoring with the automatic monitor located on the Coast Road NTC01.  This will be actioned for the start of the diffusion tube monitoring programme in 2027.

North Tyneside Council has taken forward a number of direct measures during the current reporting year of 2025 in pursuit of improving local air quality. Details of all measures completed, in progress or planned are set out in Table 2.2. Eight measures are included within Table 2.2, with the type of measure and the progress North Tyneside Council have made during the reporting year of 2025 presented. Where there have been, or continue to be, barriers restricting the implementation of the measure, these are also presented within Table 2.2.

More detail on these measures can be found in their respective Action Plans.  These eight measures are aligned with actions set out in the Air Quality Strategy. Further information is available within the relevant Action Plans associated with the Strategy, the North Tyneside Low Carbon Plan 2016–2027, Transport Strategy, Cycling Strategy, Home to School/College Transport Policy, Hackney Carriage and Private Hire Licensing Policy, Network Management Plan, and the Net Zero 2030 Action Plan. Collectively, these plans support the delivery of initiatives aimed at improving air quality.

Key completed measures are:

- Monitoring and ratification of NO2 at 29 diffusion tube locations.

- Support of real time continuous air quality monitoring for NO2 and Particulates, including PM2.5 at the real time air station located on the Coast Road A1058.

- Use of 5 indicative real time samplers on Holystone Way, A191, Mariners Lane, Tynemouth, Richardson Street, Wallsend and Whitley Lodge, Monkseaton.  These air samplers monitor NO2 and Particulates, including PM2.5, providing a guide to the air quality in the area.

- Enforcement of smoke control orders. Where complaints are received, they are fully investigated to ensure residents are burning the correct fuels for use within the smoke control areas.  Information is requested on the fuels purchased, and where wood is being burnt to ensure the appliance is on the exempt appliance list.

- Promotion of the ‘Go smarter’ scheme to encourage alternative transport modes such as walking or cycling to school.

- North Tyneside Council continues to encourage and promote alternative clean transport modes such as cycling to improve air quality and promotes a cycle to work scheme via salary sacrifice. North Tyneside’s cycling strategy is available on the Authority’s website.

- A car sharing scheme is available for all employees located at North Tyneside Councils main offices at The Quadrant, which is managed by the Cobalt Travel Centre who coordinate drivers and passengers.  The scheme supports employees to consider car sharing as a cost effective and more sustainable travel option.

North Tyneside Council expects the following measures to be completed over the course of the next reporting year:

- Co-location of NO2 tubes to the Real Time Continuous Monitoring air station located on the Coast Road A1058. Wallsend   which monitors NO2 and particulates for validation.

  - Indicative passive monitoring of NO2 at 29 diffusion tube locations located about the Borough and indicative real time monitoring for NO2 and particulates about road side and background urban locations.

  - Review of monitoring locations and air quality results and local current concerns to ensure that monitors are positioned at the most appropriate locations as per Defra’s Local Air Quality Management technical guidance (TG22).

  - Review of the five indicative real time NO2 and Particulate monitors, including PM2.5 two located about Holystone Way, A191 and three located at urban background residential areas of Wallsend, Tynemouth and Whitley Bay.

- Smoke enforcement measures including public engagement and education through awareness campaign.  Routine inspections in residential areas where there is high wood burning appliance use and distribution of guidance leaflets to households in these areas.

  - Continued expansion of the electric vehicle charging network to encourage the uptake of electric vehicles.

North Tynesideside Council worked to implement these measures in partnership with the following stakeholders during 2025:

- Public Health

- Development Control

- Local Highways

North Tyneside Council does not anticipate any challenges and barriers to the implementation of the above measures.

Table 2.1 – Progress on Measures to Improve Air Quality

| Measure No. | Measure Title | Category | Classification | Year Measure Introduced in AQAP | Estimated / Actual Completion Date | Organisations Involved | Funding Source | Funding Status | Estimated Cost of Measure | Measure Status | Reduction in Pollutant / Emission from Measure | Key Performance Indicator | Progress to Date | Comments / Barriers to Implementation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Priority Air Quality Actions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. | Net Zero Action Plan | Policy Guidance and Development Control | Other Policy | 2019 | 2030 | Local Authority | NA | NO | Not Funded | Implementation | Reduced vehicle emissions | Reduction of NO2, PM | Implementation on-going | NA |
| 2 | Encourage the use of electric vehicles by providing public charging points | Promoting Low Emission Transport | Procuring alternative Refuelling infrastructure to promote Low Emission Vehicles, EV recharging, Gas fuel recharging | 2025 | 2027 | Local Authority Transport Dept.; North East Combined Authority | Government grant funding via North East Combined Authority | Partially funded | >£500k | Development | Estimated to be less than 1µg/m µg/m3 based on a low to medium uptake | Based on measures of chargepoint usage | Development ongoing | Electricity grid capacity limitations can impact viability of locations |
| 3 | Smoke Control Enforcement | Public Information | Via other mechanisms | 2023 | 2026 | Local Authority Environmental Health | Smoke Enforcement Grant | Yes | £11k | Planned | Reduced Smoke Emissions | Reduction of PM | Implementation on-going | NA |
| 4 | School Road Closure | Promoting Travel Alternatives | Schools Travel Plans | 2022 | Ongoing | Local Authority Transport Dep. | NA | NO | Not Funded | Implementation | Reduced vehicle emissions | Reduction of NO2, PM | Implementation on-going | NA |
| 5 | Compliance Charge for Part B Processes | Environmental Permits | Other | N/A | Ongoing | Local Authority Environmental Health | NA | NO | Not Funded | Implementation | Reduced industrial emissions | Reduction of NO2, PM | Implementation on-going | NA |
| 6 | Taxi Licensing Scheme | Promoting Low Emission Transport | Taxi Licensing conditions | 2020 | Ongoing | Local Authority Licensing | NA | NO | Not Funded | Implementation | Reduced vehicle emissions | Reduction of NO2, PM | Implementation on-going | NA |
| 7 | Cobalt Car Sharing Scheme | Promoting Travel Alternatives | Workplace Travel Planning | 2015 | N/A | Cobalt Travel Team | NA | NO | < £10k | Implementation | Reduced vehicle emissions | Reduction of NO2 and PM | Implementation on-going | NA |
| 8 | Car Lease Scheme | Promoting Low Emission Transport | Company Vehicle Procurement |  | Ongoing | NTC | NA | NO | Not Funded | Implementation | Reduced Vehicle Emissions | Reduction of NO2 and PM | Implementation on-going | NA |

PM2.5 – Local Authority Approach to Reducing Emissions and/or Concentrations

As detailed in Policy Guidance LAQM.PG22 (Chapter 8) and the Air Quality Strategy, local authorities are expected to work towards reducing emissions and/or concentrations of fine particulate matter (PM2.5)). There is clear evidence that PM2.5 (particulate matter smaller 2.5 micrometres) has a significant impact on human health, including premature mortality, allergic reactions, and cardiovascular diseases.

Although PM2.5 concentrations in the borough are comparatively low, there remains scope for further improvement. Evidence from the Public Health Outcomes Framework indicates that exposure to human-made fine particulate matter has a measurable impact on life expectancy and contributes to mortality within the local population. The most recent data, reported in 2024, estimates that PM2.5 pollution accounts for approximately 4.7% of all deaths in North Tyneside, slightly below the England average of 5.3%.

Long-term exposure to poor air quality, particularly fine particulate matter (PM2.5), is widely recognised as a significant public health concern. Prolonged inhalation of these microscopic particles allows them to penetrate deep into the lungs and enter the bloodstream, where they can trigger inflammation and oxidative stress within the body. Over time, this can lead to the development or exacerbation of a range of serious health conditions.

There is strong evidence linking long-term exposure to elevated PM2.5 concentrations with an increased risk of cardiovascular diseases, including heart attacks, strokes, hypertension, and atherosclerosis. Fine particulates contribute to the narrowing and hardening of arteries, impair normal heart function, and increase the likelihood of life-threatening cardiovascular events. Individuals with pre-existing heart conditions, as well as older adults, are particularly vulnerable to these effects.

In addition, there is well-established evidence of a causal relationship between prolonged exposure to particulate air pollution and lung cancer. PM2.5 particles often carry harmful substances, including carcinogens, which can damage lung tissue and DNA over time. This increases the likelihood of cancer development, even among non-smokers.

Levels of PM 2.5 in North Tyneside as measured by the Coast Road NT01 automatic monitoring station are generally low.  The annual mean results for particulates PM2.5 for 2025 at Wallsend roadside site was 8.6μg/m3 which is well below the EU target of 25 μg/m3 and UK target of 20 μg/m3.  Actions already taken by North Tyneside Council to reduce pollutants such as PM10 and NO2 as reported in Table 2.1 will support the reduction of PM2.5 emissions.

North Tyneside Council is taking the following existing measures to address PM2.5:

- North Tyneside Council currently monitors PM2.5 at one real time station within the borough, located on the Coast Road A1058.  The PM2.5 pollutant levels at this location have shown a slight decrease over the last two years. This is considered to be the result of a temporary reduction in the speed limit for traffic travelling along the main Coast Road.

- Indicative air samplers are utilised in 5 monitoring locations; these provide an indication air quality for PM2.5 and PM10 pollutant levels.  These were introduced mid-2024 and 2025.  The indicative air samplers have shown that pollutant levels for both NO2, PM2.5 and PM10 are well below the annual mean objective levels.

- There has been no further update provided on the proposed government Automatic Urban and Rural Network (AURN) PM2.5 monitor to be installed to measure the urban background emissions within North Shields. The timescale for its installation is still unknown; however, such monitoring will be valuable for supporting the local air quality monitoring currently carried out.

- An Air Quality steering group operates and meets to prioritise actions and measures to tackle PM2.5. The membership consists of all relevant partners including Transport, Planners, Public Health, Planning, Sustainability and Environmental Health.

- An Air Quality Strategy has been adopted that sets effective goals to bring about air quality improvements.  An update on the progress of meeting the aims and actions of the air quality strategy is provided in Section 2.1.

- Air quality inspections and enforcement around permitted polluting industrial processes are carried out in line with government requirements.

- Education and enforcement of smoke legislation is undertaken to ensure compliance with smoke legislation around domestic solid fuel burning.  An air quality grant of £12,000 was received which was utilised for proactive enforcement of the smoke control orders. In 2025 ten warning letters were issued to residents following complaints about visible smoke emissions. The warning letters include for advice on authorised fuels, exempt appliances for use within smoke control orders and the Ready to Burn scheme.

- An ongoing commitment to bring about traffic management improvements to reduce congestion.

- Reduce emissions from new developments (during the construction phase and in subsequent use), and existing buildings by implementing energy efficiency measures and affordable warmth schemes to reduce heat loss and drive down fuel bills.

- Engagement with new developments during the construction phase and working with the industry to ensure the implementation of adequate dust suppression and logistics around vehicular movement and the management of materials on site.

- Encouraging the use of public transport and promoting alternative modes of transport to reduce air quality emissions from vehicles. Promote and encourage the uptake of alternative ‘low emission’ vehicles within the Authority. Planning requirements to require travel plans for new developments. Building approval documents requiring electric charging points. Other incentives to reduce pollution by parking charges, and reducing of engines idling etc.

- Promotion of the Cycling Strategy. The Authority promotes and encourages cycling as a healthy and sustainable way of making every day journeys. The Cycling Strategy outlines North Tyneside Council’s strategic approach to supporting cycling in the Borough.

North Tyneside Council remains committed to improving awareness and understanding of air pollution among residents. This will be achieved through the continued provision of clear, accessible, and up-to-date information on air quality. Public engagement will focus on promoting practical measures that individuals can take to reduce their exposure and contribution to air pollution, alongside raising awareness of associated health impacts. This information will be maintained and regularly updated via the Authority’s dedicated air quality webpage.

The majority of North Tyneside falls within designated Smoke Control Areas, where restrictions are in place to limit emissions from the burning of solid fuels. In response to growing concerns regarding particulate emissions from domestic solid fuel use, the Authority has secured grant funding from DEFRA which will be utilised to support enforcement of the smoke control orders.  The Authority has three indicative real-time air samplers monitoring particulate monitors located in residential locations within Whitley Bay, Tynemouth, and Wallsend. These monitors are being utilised to provide insight into trends and concentrations of particulates associated with increased solid fuel use.

Road traffic emissions continue to represent one of the principal sources of fine particulate matter (PM2.5) within the borough. These emissions arise not only from engine exhausts but also from non-exhaust sources such as tyre wear, brake abrasion, and road surface degradation. The Authority is therefore prioritising a range of measures aimed at reducing vehicle use and congestion, as well as encouraging both residents and businesses to transition towards cleaner and alternative fuel vehicles. These interventions are expected to contribute to a measurable reduction in PM2.5 concentrations.

As part of this approach, the Authority has implemented strengthened emissions standards for licensed vehicles. New taxi vehicle licences will not be granted unless the vehicle meets minimum emissions standards of Euro 6 for diesel engines and Euro 5 for petrol engines. Furthermore, from 1 April 2026, existing vehicle licences are not being renewed unless these standards are met. This requirement will be extended to wheelchair-accessible vehicles from 1 April 2027, ensuring a consistent and progressive improvement in the emissions performance of the licensed fleet.

The Authority also addresses particulate emissions arising from construction activities. All major development proposals are required to include a Construction Environmental Management Plan (CEMP) as part of the planning approval process. This ensures that appropriate mitigation measures are in place to minimise dust emissions during construction and demolition phases, recognising the potential for such activities to contribute to localised air quality impacts. In addition, any complaints relating to dust are thoroughly investigated, and Environmental Health officers undertake monitoring and enforcement where necessary. Industrial installations are similarly subject to ongoing scrutiny to ensure effective control of emissions.

Industrial emissions within the borough are regulated under the Environmental Permitting (England and Wales) Regulations 2016. Businesses that meet specified thresholds are required to obtain either a Part B or A2 permit, issued and regulated by North Tyneside Council. During 2025, one new Part B permit was granted for a vehicle vapour recovery process for the storage and handling of petrol, and the control of petrol vapour emissions during deliveries and vehicle refuelling. These processes can release volatile organic compounds (VOCs), which contribute to air pollution and ozone formation. The Part B permit was required to cover for the petrol unloading from tankers into underground storage tanks (Stage I vapour recovery) and for the refuelling of vehicles at pumps (Stage II vapour recovery), as the throughput exceeds the thresholds where a permit is required.

All permitted installations are inspected in accordance with DEFRA guidance to ensure compliance with national process guidance note emission limits. The Authority works to ensure that emissions are reduced to the lowest practicable level, taking into account both technical feasibility and cost, and that they do not give rise to significant pollution.

# Air Quality Monitoring Data and Comparison with Air Quality Objectives and National Compliance

This section sets out the monitoring undertaken within 2025 by North Tyneside Council and how it compares with the relevant air quality objectives. In addition, monitoring results are presented for a five-year period between 2021 and 2025 to allow monitoring trends to be identified and discussed.

Summary of Monitoring Undertaken

### Automatic Monitoring Sites

North Tyneside Council undertook automatic (continuous) monitoring at 1 site during 2025. Table A.1 in Appendix A shows the details of the automatic monitoring sites. NB. Local authorities do not have to report annually on the following pollutants: 1,3 butadiene, benzene, carbon monoxide and lead, unless local circumstances indicate there is a problem. Historic monitoring for benzene did not determine any concerns over this pollutant level and monitoring is no longer carried out. The Air Quality England website page presents automatic monitoring results for North Tyneside Council, with automatic monitoring results also available through the UK-Air website .

Maps showing the location of the monitoring sites are provided in Appendix D. Further details on how the monitors are calibrated and how the data has been adjusted are included in Appendix C.

The results of ratified adjusted data for NO2, PM10 and PM2.5 is provided in Appendix A tables A.3, A.5, A.6, A.7, and A.8, and the results are discussed in section 3.2.

### Non-Automatic Monitoring Sites

North Tyneside Council undertook non- automatic (i.e. passive) monitoring of NO2 at 29 sites during 2025. Table A.2 in Appendix A presents the details of the non-automatic sites.

Maps showing the location of the monitoring sites are provided in Appendix D. Further details on Quality Assurance/Quality Control (QA/QC) for the diffusion tubes, including bias adjustments and any other adjustments applied (e.g. annualisation and/or distance correction), are included in Appendix C.

There were no new diffusion tube monitoring sites introduced in 2025.

Individual Pollutants

The air quality monitoring results presented in this section are, where relevant, adjusted for bias, annualisation (where the annual mean data capture is below 75% and greater than 25%), and distance correction. Further details on adjustments are provided in Appendix C.

### Nitrogen Dioxide (NO2)

Table A.3 and Table A.4 in Appendix A compare the ratified and adjusted monitored NO2 annual mean concentrations for the past five years with the air quality objective of 40µg/m3. Note that the concentration data presented represents the concentration at the location of the monitoring site, following the application of bias adjustment and annualisation, as required (i.e. the values are exclusive of any consideration to fall-off with distance adjustment).

For diffusion tubes, the full 2025 dataset of monthly mean values is provided in Appendix B. Note that the concentration data presented in Table B.1 includes distance corrected values, only where relevant.

Table A.5 in Appendix A compares the ratified continuous monitored NO2 hourly mean concentrations for the past five years with the air quality objective of 200µg/m3, not to be exceeded more than 18 times per year.

The indicative NO2 diffusion tube monitoring in North Tyneside all had annual mean levels below the annual mean objective in 2025 following bias adjustment. There was no requirement for any of the monitoring sites to have distance correction. Annualisation of the monitoring data was required for 1 site CH1 located in Norham Road/Rothbury Terrace near Percy Main. None of the indicative NO2 diffusion tube monitoring locations have pollutant concentrations within 10% of the annual mean objective level and the annual mean NO2 pollutant levels do not suggest any indication that concentrations will have exceedances of the 1-hour NO2 mean objective level.

The automatic monitoring data in Wallsend gave an annual mean NO2 concentration of 28.3 μg/m3 which is below the annual mean objective level of 40 μg/m3.  The 2025 concentration level is slightly higher than the 2024 but is still below the level of 32.2 μg/m3  monitored in 2023. This is likely due to increased traffic levels operating along the Coast Road A1058.

The long-term 5 year Annual Mean NO2 trend chart in Figure A.1 provides a comparison between 2021 and 2025 of the long-term NO2 indicative monitoring sites in the borough of annual means.  The chart shows a slight increase in NO2 during 2025 compared to 2024, but the trend shows that all long-term monitoring sites are well below the annual mean objective level of 40 µg/m3.  The NO2 monitoring data shows that the Coast Road real time station recorded the highest annual mean concentration at 28.3 μg/m3  given in table A.3. The Coast Road roadside monitoring location is adjacent to one of the main arterial roads into the borough, with high traffic volumes.  The next highest concentration was at the monitoring location CH1 located on Norham Road/Rothbury Terrace in West Chirton.  This site is roadside and is adjacent to a bus depot and next to a row of terraced properties.  An annual mean concentration of 25.0 μg/m3 was recorded at this monitoring site.  The lowest annual mean concentration of 10.6 μg/m3   was recorded at KW1, which is located on Moorfield Drive, Killingworth. KW1 was also the monitoring location with the lowest annual mean concentration reported in the 2025 air quality status report.

The results for 2025 suggest that there is no requirement to declare any AQMAs in North Tyneside.

### Particulate Matter (PM10)

Table A.6 in Appendix A: Monitoring Results compares the ratified and adjusted monitored PM10 annual mean concentrations for the past five years with the air quality objective of 40µg/m3.

Table A.7 in Appendix A compares the ratified continuous monitored PM10 daily mean concentrations for the past five years with the air quality objective of 50µg/m3, not to be exceeded more than 35 times per year.

North Tyneside Council undertook particulate (PM10) monitoring during 2025 at the real-time monitoring station located on the A1058 Coast Road. Measured concentrations remained well below the air quality objective, consistent with trends observed over the preceding five years. The annual mean objective was therefore not exceeded at this monitoring location.

Data from 2025 indicates that there were eight instances where the 24-hour mean concentration exceeded 50 µg/m³. Investigation identified that these exceedances were associated with tree clearance activities at Wallsend Golf Club. The club had obtained the appropriate permit for the burning of wood waste; however, frequent and substantial bonfires were undertaken to the north of the site, directly in line with the monitoring station, resulting in short-term elevated particulate levels.

The long-term (five-year) trend in annual mean PM10 concentrations between 2020 and 2025 is illustrated in Figure A.2. Figure A.2 indicates a slight reduction in the annual mean for PM10 over the 5 year period, indicating a gradual improvement in particulate levels at this location.

The results for 2025 for PM10 suggest that there is no requirement to declare any AQMAs in North Tyneside.

### Particulate Matter (PM2.5)

PM2.5 monitoring is carried out at the real time monitoring site located on the Coast Road A1058. Table A.8 in Appendix A presents the ratified and adjusted monitored PM2.5 annual mean concentrations for the past five years.

The annual mean concentration for PM2.5 in 2025 was 8.6 µg/m3, which was an increase in the concentration mean compared to 2024.

# Appendix A: Monitoring Results

Table A.1 – Details of Automatic Monitoring Sites

| Site ID | Site Name | Site Type | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Pollutants Monitored | In AQMA? | Which AQMA? (1) | Monitoring Technique | Distance to Relevant Exposure (m) (1,2) | Distance to kerb of nearest road (m) (1) | Inlet Height (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | Coast Road | Roadside | 428352 | 566974 | NO2, PM10, PM2.5 | No | N/A | Chemiluminescent; Palas FIDAS (optical light scattering) | 37.0 | 2.0 | 2.0 |

Notes:

(1) N/A if not applicable

(2) 0m if the monitoring site is at a location of exposure (e.g. installed on the façade of a residential property).

Table A.2 – Details of Non-Automatic Monitoring Sites

| Diffusion Tube ID | Site Name | Site Type | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Pollutants Monitored | In AQMA? Which AQMA? | Distance to Relevant Exposure (m) (1) | Distance to kerb of nearest road (m) (2) | Tube Co-located with a Continuous Analyser? | Tube Height (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM1 | Park Lane, Shiremoor | Roadside | 431743 | 570649 | NO2 | NO | 4.0 | 2.0 | NO | 3.0 |
| BR1 | Burradon road, Annitsford | Roadside | 427012 | 573655 | NO2 | NO | 5.0 | 2.0 | NO | 3.0 |
| CC1 | John Street, Cullercoats | Kerbside | 436246 | 571385 | NO2 | NO | 8.0 | <1 | NO | 3.0 |
| CH1 | Norham Road/ Rothbury Terrace | Kerbside | 433580 | 567865 | NO2 | NO | N/A | <1 | NO | 3.0 |
| CM1 | Broadway, Cullercoats | Roadside | 435205 | 571823 | NO2 | NO | 15.0 | <1 | NO | 3.0 |
| ER1 | Oxen Drive, Benton | Roadside | 428965 | 568813 | NO2 | NO | 19.0 | 2.0 | NO | 3.0 |
| GH1 | Lower Crane Street, Shiremoor | Kerbside | 431749 | 571815 | NO2 | NO | 5.0 | 2.0 | NO | 3.0 |
| HR1 | Bewicke Road, Willington Quay | Suburban | 432683 | 566400 | NO2 | NO | 5.0 | 2.0 | NO | 3.0 |
| HW3 | Meldon Street, East Howdon | Roadside | 433194 | 566418 | NO2 | NO | 2.0 | <1 | NO | 3.0 |
| KW1 | Moorfield Drive,Killingworth | Roadside | 428784 | 570855 | NO2 | NO | 10.0 | 2.0 | NO | 3.0 |
| LB1 | West Farm Avenue, Benton | Roadside | 426795 | 568772 | NO2 | NO | 7.0 | 2.0 | NO | 3.0 |
| LB2 | Front Street/Benton Road, Longbenton | Kerbside | 427079 | 568377 | NO2 | NO | 2.0 | <1 | NO | 3.0 |
| LH7 | Battlehill Drive, Wallsend | Roadside | 430713 | 567967 | NO2 | NO | 4.0 | 4.0 | NO | 3.0 |
| LP1 | Dudley Lane, Seaton Burn | Kerbside | 424233 | 573771 | NO2 | NO | 10.0 | <1 | NO | 3.0 |
| MC1 | White House Drive, Wide Open | Kerbside | 426308 | 570958 | NO2 | NO | N/A | 2.0 | NO | 3.0 |
| MR1 | Manor Road, Tynemouth | Roadside | 436722 | 569438 | NO2 | NO | 10.0 | <1 | NO | 3.0 |
| MT1 | New York Road | Roadside | 432925 | 570024 | NO2 | NO | 17.0 | 2.0 | NO | 3.0 |
| MT2 | Rake Lane | Roadside | 433776 | 570239 | NO2 | NO | 24.0 | 2.0 | NO | 3.0 |
| NS10 | Queen Alexandra Road, North Shields | Kerbside | 434093 | 569102 | NO2 | NO | NA | <1 | NO | 3.0 |
| PG2 | North Road, Preston Road, Preston Grange | Roadside | 435069 | 569860 | NO2 | NO | 1.0 | 2.0 | NO | 3.0 |
| RV1 | Monkseaton Drive, Whitley Bay | Roadside | 435076 | 573325 | NO2 | NO | 5.0 | 2.0 | NO | 3.0 |
| SP1 | Holystone Way, Holystone | Roadside | 430444 | 570242 | NO2 | NO | 5.0 | 2.0 | NO | 3.0 |
| TR1 | Tynemouth Road, Rosehill | Roadside | 431808 | 566954 | NO2 | NO | 3.0 | 2.0 | NO | 3.0 |
| TY1 | Front Street, Tynemouth | Roadside | 437028 | 569379 | NO2 | NO | 3.0 | <1 | NO | 3.0 |
| W10 | Coast Road, Wallsend | Kerbside | 429331 | 567395 | NO2 | NO | 30.0 | 2.0 | NO | 3.0 |
| W99 | Frank Street, Wallsend | Kerbside | 429951 | 566324 | NO2 | NO | 2.0 | 2.0 | NO | 3.0 |
| WB20 | Grosvenor Drive/Norham Road, Whitley Bay | Kerbside | 434955 | 572037 | NO2 | NO | N/A | <1 | NO | 3.0 |
| WM1 | Great Lime Road, Killingworth | Roadside | 426738 | 570543 | NO2 | NO | 13.0 | 2.0 | NO | 3.0 |
| WR1 | Whitley Road, Whitley Bay | Kerbside | 435767 | 572041 | NO2 | NO | 10.0 | 2.0 | NO | 3.0 |

Notes:

(1) 0m if the monitoring site is at a location of exposure (e.g. installed on the façade of a residential property).

(2) N/A if not applicable.

Table A.3 – Annual Mean NO2 Monitoring Results: Automatic Monitoring (µg/m3)

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | 428352 | 566974 | Roadside | 92.1 | 92.1 | 35.9 | 35.4 | 32.3 | 24.4 | 28.3 |

☒ Annualisation has been conducted where data capture is <75% and >25% in line with LAQM.TG22.

☒ Reported concentrations are those at the location of the monitoring site (annualised, as required), i.e. prior to any fall-off with distance correction.

☒ Where exceedances of the NO2 annual mean objective occur at locations not representative of relevant exposure, the fall-off with distance concentration has been calculated and reported concentration provided in brackets for 2025.

Notes:

The annual mean concentrations are presented as µg/m3.

Exceedances of the NO2 annual mean objective of 40µg/m3 are shown in bold.

All means have been “annualised” as per LAQM.TG22 if valid data capture for the full calendar year is less than 75%. See Appendix C for details.

Concentrations are those at the location of monitoring and not those following any fall-off with distance adjustment.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

Table A.4 – Annual Mean NO2 Monitoring Results: Non-Automatic Monitoring (µg/m3)

| Diffusion Tube ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM1 | 431743 | 570649 | Roadside | 100.0 | 100.0 | 16.6 | 16.9 | 16.0 | 14.8 | 16.2 |
| BR1 | 427012 | 573631 | Roadside | 100.0 | 100.0 | 11.0 | 13.2 | 12.2 | 11.0 | 11.3 |
| CC1 | 436246 | 571385 | Kerbside | 100.0 | 100.0 | NA | 14.0 | 12.1 | 11.1 | 11.7 |
| CH1 | 433580 | 567865 | Kerbside | 67.3 | 67.3 | 22.1 | 25.0 | 20.9 | 21.0 | 25.0 |
| CM1 | 435814 | 571093 | Industrial | 100.0 | 100.0 | 12.2 | 13.5 | 11.8 | 10.9 | 11.1 |
| ER1 | 428960 | 568813 | Roadside | 100.0 | 100.0 | NA | NA | NA | 14.2 | 14.0 |
| GH1 | 431750 | 571818 | Kerbside | 100.0 | 100.0 | 17.5 | 18.5 | 16.3 | 14.8 | 15.0 |
| HR1 | 432693 | 566393 | Suburban | 100.0 | 100.0 | 19.5 | 23.6 | 20.5 | 18.1 | 18.7 |
| HW3 | 433199 | 566428 | Roadside | 100.0 | 100.0 | 16.7 | 18.4 | 16.1 | 14.5 | 14.4 |
| KW1 | 428784 | 570855 | Roadside | 100.0 | 100.0 | NA | NA | NA | 9.4 | 10.6 |
| LB1 | 426795 | 568772 | Kerbside | 100.0 | 100.0 | NA | NA | NA | 18.5 | 19.2 |
| LB2 | 427067 | 568371 | Kerbside | 100.0 | 100.0 | 18.8 | 20.5 | 21.6 | 21.4 | 20.9 |
| LH7 | 430713 | 567967 | Roadside | 100.0 | 100.0 | 20.0 | 23.5 | 23.0 | 21.5 | 20.7 |
| LP1 | 424207 | 573779 | Kerbside | 100.0 | 100.0 | 12.8 | 10.2 | 12.4 | 11.9 | 12.1 |
| MC1 | 426306 | 570959 | Kerbside | 92.3 | 92.3 | 15.2 | 17.1 | 16.7 | 15.8 | 15.8 |
| MR1 | 436724 | 569441 | Kerbside | 100.0 | 100.0 | 12.3 | 13.2 | 11.8 | 11.6 | 12.2 |
| MT1 | 432925 | 570024 | Roadside | 84.6 | 84.6 | NA | NA | NA | 13.1 | 13.2 |
| MT2 | 433776 | 570239 | Roadside | 75.0 | 75.0 | NA | NA | NA | 18.6 | 17.2 |
| NS10 | 434096 | 569100 | Kerbside | 100.0 | 100.0 | 19.2 | 20.0 | 17.1 | 16.6 | 16.7 |
| PG2 | 435070 | 569855 | Roadside | 84.6 | 84.6 | 20.3 | 22.1 | 17.2 | 14.5 | 15.3 |
| RV1 | 435076 | 573325 | Roadside | 100.0 | 100.0 | NA | 18.1 | 15.3 | 13.6 | 14.6 |
| SP1 | 430444 | 570242 | Roadside | 100.0 | 100.0 | 21.5 | 23.0 | 21.1 | 19.0 | 19.3 |
| TR1 | 431747 | 566956 | Roadside | 100.0 | 100.0 | 21.2 | 24.0 | 21.8 | 17.9 | 19.8 |
| TY1 | 437019 | 569375 | Roadside | 100.0 | 100.0 | 18.1 | 18.8 | 17.3 | 14.6 | 14.9 |
| W10 | 429262 | 567378 | Kerbside | 100.0 | 100.0 | 23.1 | 26.1 | 22.2 | 21.6 | 19.8 |
| W99 | 429941 | 566319 | Kerbside | 100.0 | 100.0 | 19.0 | 20.2 | 16.9 | 15.3 | 15.2 |
| WB20 | 435411 | 571995 | Kerbside | 100.0 | 100.0 | 13.1 | 13.2 | 16.6 | 10.6 | 11.1 |
| WM1 | 426738 | 570541 | Roadside | 100.0 | 100.0 | NA | NA | NA | 13.9 | 15.1 |
| WR1 | 435797 | 572026 | Kerbside | 100.0 | 100.0 | 17.0 | 16.9 | 14.7 | 13.9 | 15.4 |

☒ Annualisation has been conducted where data capture is <75% and >25% in line with LAQM.TG22.

☒ Diffusion tube data has been bias adjusted.

☒ Reported concentrations are those at the location of the monitoring site (bias adjusted and annualised, as required), i.e. prior to any fall-off with distance correction.

Notes:

The annual mean concentrations are presented as µg/m3.

Exceedances of the NO2 annual mean objective of 40µg/m3 are shown in bold.

NO2 annual means exceeding 60µg/m3, indicating a potential exceedance of the NO2 1-hour mean objective are shown in bold and underlined.

Means for diffusion tubes have been corrected for bias. All means have been “annualised” as per LAQM.TG22 if valid data capture for the full calendar year is less than 75%. See Appendix C for details.

Concentrations are those at the location of monitoring and not those following any fall-off with distance adjustment.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

Figure A.1 – Trends in Annual Mean NO2 Concentrations

Table A.5 – 1-Hour Mean NO2 Monitoring Results, Number of 1-Hour Means > 200µg/m3

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | 428352 | 566974 | Roadside | 92.1 | 92.1 | 0 | 0 | 0 | 0 | 0 |

Notes:

Results are presented as the number of 1-hour periods where concentrations greater than 200µg/m3 have been recorded.

Exceedances of the NO2 1-hour mean objective (200µg/m3 not to be exceeded more than 18 times/year) are shown in bold.

If the period of valid data is less than 85%, the 99.8th percentile of 1-hour means is provided in brackets.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

Table A.6 – Annual Mean PM10 Monitoring Results (µg/m3)

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | 428352 | 566974 | Roadside | 89.8 | 89.8 | 16.7 | 20.9 | 19.7 | 16.6 | 18.4 |

☒ Annualisation has been conducted where data capture is <75% and >25% in line with LAQM.TG22.

Notes:

The annual mean concentrations are presented as µg/m3.

Exceedances of the PM10 annual mean objective of 40µg/m3 are shown in bold.

All means have been “annualised” as per LAQM.TG22 if valid data capture for the full calendar year is less than 75%. See Appendix C for details.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

Figure A.2 – Trends in Annual Mean PM10 Concentrations at Automatic Monitoring Location NTC01

Table A.7 – 24-Hour Mean PM10 Monitoring Results, Number of PM10 24-Hour Means > 50µg/m3

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | 428352 | 566974 | Roadside | 89.8 | 89.8 | 1 | 8 | 9 | 0 (35) | 8 |

Notes:

Results are presented as the number of 24-hour periods where daily mean concentrations greater than 50µg/m3 have been recorded.

Exceedances of the PM10 24-hour mean objective (50µg/m3 not to be exceeded more than 35 times/year) are shown in bold.

If the period of valid data is less than 85%, the 90.4th percentile of 24-hour means is provided in brackets.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

Table A.8 – Annual Mean PM2.5 Monitoring Results (µg/m3)

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Site Type | Valid Data Capture for Monitoring Period (%) (1) | Valid Data Capture 2025 (%) (2) | 2021 | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTC01 | 428352 | 566974 | Roadside | 89.8 | 89.8 | 8 | 9 | 8.5 | 7.7 | 8.6 |

☒ Annualisation has been conducted where data capture is <75% and >25% in line with LAQM.TG22.

Notes:

The annual mean concentrations are presented as µg/m3.

All means have been “annualised” as per LAQM.TG22 if valid data capture for the full calendar year is less than 75%. See Appendix C for details.

(1) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(2) Data capture for the full calendar year (e.g. if monitoring was carried out for 6 months, the maximum data capture for the full calendar year is 50%).

# Appendix B: Full Monthly Diffusion Tube Results for 2025

Table B.1 – NO2 2025 Diffusion Tube Results (µg/m3)

| DT ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean: Raw Data | Annual Mean: Annualised and Bias Adjusted <(0.89)> | Annual Mean: Distance Corrected to Nearest Exposure | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM1 | 431743 | 570649 | 32.4 | 11.0 | 20.6 | 16.0 | 11.9 | 12.2 | 10.0 | 16.3 | 19.6 | 17.1 | 30.9 | 19.9 | 18.2 | 16.2 |  |  |
| BR1 | 427012 | 573631 | 20.7 | 0.3 | 17.9 | 13.3 | 9.5 | 8.8 | 8.5 | 12.7 | 14.3 | 11.6 | 19.7 | 15.7 | 12.7 | 11.3 |  |  |
| CC1 | 436246 | 571385 | 26.3 | 14.4 | 14.5 | 9.4 | 7.7 | 8.3 | 7.1 | 9.9 | 12.4 | 13.4 | 18.9 | 15.2 | 13.1 | 11.7 |  |  |
| CH1 | 433580 | 567865 | N/A | 50.5 | 31.5 | 27.4 | 20.7 | 18.1 | N/S |  | n/s | 24.2 | 35.2 | 25.4 | 29.1 | 25.0 |  |  |
| CM1 | 435814 | 571093 | 24.7 | 7.7 | 16.7 | 12.7 | 7.7 | 7.7 | 7.6 | 10.5 | 12.2 | 11.4 | 18.1 | 13.4 | 12.5 | 11.1 |  |  |
| ER1 | 428960 | 568813 | 26.9 | 9.9 | 19.5 | 13.4 | 11.1 | 12.1 | 8.9 | 12.9 | 16.7 | 15.4 | 23.9 | 17.5 | 15.7 | 14.0 |  |  |
| GH1 | 431750 | 571818 | 30.1 | 18.3 | 9.4 | 14.4 | 11.4 | 13.4 | 11.3 | 14.8 | 18.8 | 16.9 | 24.6 | 19.1 | 16.9 | 15.0 |  |  |
| HR1 | 432693 | 566393 | 31.1 | 12.2 | 24.8 | 24.8 | 17.2 | 18.3 | 15.1 | 19.5 | 21.8 | 18.6 | 26.1 | 22.3 | 21.0 | 18.7 |  |  |
| HW3 | 433199 | 566428 | 28.2 | 8.8 | 21.7 | 15.4 | 11.2 | 11.4 | 10.7 | 12.7 | 16.3 | 15.5 | 23.4 | 18.2 | 16.1 | 14.4 |  |  |
| KW1 | 428784 | 570855 | 22.8 | 8.1 | 15.7 | 11.9 | 7.5 | 7.1 | 6.8 | 8.7 | 11.9 | 10.5 | 19.1 | 12.8 | 11.9 | 10.6 |  |  |
| LB1 | 426795 | 568772 | 34.5 | 12.7 | 27.9 | 22.4 | 14.9 | 16.4 | 16.2 | 18.5 | 20.2 | 24.2 | 25.8 | 24.9 | 21.5 | 19.2 |  |  |
| LB2 | 427067 | 568371 | 37.9 | 16.5 | 29.0 | 23.1 | 17.6 | 20.4 | 19.0 | 21.1 | 23.1 | 20.4 | 31.7 | 22.6 | 23.5 | 20.9 |  |  |
| LH7 | 430713 | 567967 | 34.6 | 14.7 | 27.9 | 25.4 | 18.0 | 19.4 | 18.7 | 22.1 | 24.0 | 19.9 | 30.0 | 24.0 | 23.2 | 20.7 |  |  |
| LP1 | 424207 | 573779 | 22.5 | 8.3 | 15.5 | 12.5 | 9.4 | 10.0 | 9.6 | 11.8 | 14.8 | 13.6 | 19.7 | 15.3 | 13.6 | 12.1 |  |  |
| MC1 | 426306 | 570959 | 30.1 | 9.8 | 21.6 | 14.5 | 10.6 | 12.7 | 12.1 | N/S | 17.3 | 17.5 | 28.2 | 20.7 | 17.7 | 15.8 |  |  |
| MR1 | 436724 | 569441 | 25.3 | 8.6 | 15.1 | 12.7 | 8.3 | 9.8 | 7.8 | 11.7 | 15.1 | 13.6 | 21.8 | 14.9 | 13.7 | 12.2 |  |  |
| MT1 | 432925 | 570024 | 26.0 | 10.6 | 17.4 | 13.9 | 10.4 | 10.8 | 9.7 | N/S | n/s | 14.3 | 25.1 | 9.8 | 14.8 | 13.2 |  |  |
| MT2 | 433776 | 570239 | 32.3 | 11.6 | 23.1 | 20.2 | 18.9 | 16.7 | 15.6 | N/S | n/s | n/s | 25.4 | 10.5 | 19.4 | 17.2 |  |  |
| NS10 | 434096 | 569100 | 34.1 | 10.4 | 20.4 | 15.7 | 12.9 | 18.2 | 12.9 | 14.9 | 21.1 | 20.5 | 23.3 | 20.4 | 18.7 | 16.7 |  |  |
| PG2 | 435070 | 569855 | N/A | N/S | 20.2 | 15.1 | 14.8 | 14.4 | 11.1 | 13.9 | 17.7 | 18.3 | 26.7 | 19.4 | 17.2 | 15.3 |  |  |
| RV1 | 435076 | 573325 | 27.8 | 17.9 | 20.0 | 14.2 | 13.0 | 11.2 | 11.8 | 14.5 | 14.7 | 15.9 | 19.5 | 16.2 | 16.4 | 14.6 |  |  |
| SP1 | 430444 | 570242 | 29.8 | 13.3 | 29.2 | 22.7 | 20.5 | 16.0 | 16.0 | 20.6 | 22.4 | 19.2 | 28.3 | 22.2 | 21.7 | 19.3 |  |  |
| TR1 | 431747 | 566956 | 29.7 | 12.6 | 30.6 | 24.4 | 19.9 | 18.3 | 17.5 | 19.5 | 21.2 | 21.7 | 28.2 | 22.8 | 22.2 | 19.8 |  |  |
| TY1 | 437019 | 569375 | 32.7 | 9.8 | 18.7 | 15.2 | 11.1 | 14.5 | 8.7 | 14.1 | 16.8 | 16.4 | 23.5 | 19.9 | 16.8 | 14.9 |  |  |
| W10 | 429262 | 567378 | 12.3 | 15.6 | 28.0 | 24.6 | 16.5 | 23.1 | 19.6 | 23.3 | 25.6 | 22.6 | 31.3 | 24.1 | 22.2 | 19.8 |  |  |
| W99 | 429941 | 566319 | 28.5 | 9.5 | 23.9 | 16.9 | 12.8 | 11.8 | 12.7 | 15.3 | 17.5 | 17.2 | 21.9 | 17.3 | 17.1 | 15.2 |  |  |
| WB20 | 435411 | 571995 | 22.4 | 15.0 | 13.8 | 10.5 | 7.0 | 8.2 | 6.0 | 7.9 | 11.4 | 13.1 | 18.0 | 15.7 | 12.4 | 11.1 |  |  |
| WM1 | 426738 | 570541 | 26.6 | 10.3 | 20.7 | 17.1 | 11.3 | 12.4 | 11.1 | 15.7 | 17.7 | 16.1 | 26.0 | 18.5 | 17.0 | 15.1 |  |  |
| WR1 | 435797 | 572026 | 29.0 | 18.5 | 19.8 | 15.3 | 10.0 | 12.9 | 10.9 | 13.8 | 17.4 | 17.0 | 22.3 | 20.5 | 17.3 | 15.4 |  |  |

☒ All erroneous data has been removed from the NO2 diffusion tube dataset presented in Table B.1.

☒ Annualisation has been conducted where data capture is <75% and >25% in line with LAQM.TG22.

☐ Local bias adjustment factor used.

☒ National bias adjustment factor used.

☒ Where applicable, data has been distance corrected for relevant exposure in the final column.

☒ North Tyneside confirm that all 2025 diffusion tube data has been uploaded to the Diffusion Tube Data Entry System.

Notes:

Exceedances of the NO2 annual mean objective of 40µg/m3 are shown in bold.

NO2 annual means exceeding 60µg/m3, indicating a potential exceedance of the NO2 1-hour mean objective are shown in bold and underlined.

See Appendix C for details on bias adjustment and annualisation.

# Appendix C: Supporting Technical Information / Air Quality Monitoring Data QA/QC

## New or Changed Sources Identified Within North Tyneside Council During 2025

There were five new sources relating to air quality within the reporting year of 2025, where air quality has been considered. Details of the new sources are provided below.

Planning:

Four planning applications were considered in 2025 that were identified as having a potential to impact air quality.

Planning Application 24/01678/FULM

This application was for the removal of existing cooling plant and diesel generators and the installation of 15no. modular units containing plant rooms with generators and chillers stacked above.  An air quality assessment was provided that considered construction dusts which would be considered to give rise to negligible impacts.  A detailed assessment was not carried out as it was considered that an environmental permit from the Environment Agency would be required as the combined thermal input of the generators would exceed 50MW and their operation would need to meet emission limits stipulated within the permit. A condition was attached to require a detailed assessment to ensure the air quality objectives are met.

Planning Application 25/00974/FUL

This application was for the erection of 8 self-contained C2 residential flats for adults and all associated parking and infrastructure works at land at 3 Park Drive in Forest Hall.  An air quality assessment was provided with the application that considered impacts arising from the construction phase and from the operational phase.  For the construction phase the air quality impacts were assessed as not being significant if a construction management plan was implemented for mitigating dusts.  For the operational phase the development was not considered to generate more than 500 AADT or greater than 100 AADT and therefore no risk of significant impacts.

Planning Application 25/01213/FULM

This application was for the development of 82 residential plots in Backworth. An air quality assessment has considered the potential impact arising from road traffic related emissions from the A19 road affecting the proposed development. The principal pollutants of concern identified from road traffic was nitrogen dioxide and particulates. The air quality assessment concluded that the overall impact from both nitrogen dioxide and particulates would not be significant and the overall air pollutant levels would be below the air quality objective levels for NO2 and PM10 if the development was to occur.  The construction phase of the development was also assessed and impacts considered to be not significant with the implementation of good site practices and mitigation measures

Planning Application 25/01140/FULM

This planning application was for the erection of four industrial starter units for classes Egii; Egiii; B2; B8; 2 No. Class E drive thru units; 1 No. Class E unit; EV charging hub and a self-service car wash. This application is still under consideration and has not been granted planning consent as the air pollution impacts for the operational phase of the development still need to be assessed.  The air quality assessment provided with the application considered impacts arising from the construction phase but had not considered the operational phase as the traffic data was not available.  For the construction phase this was assessed as not significant if a construction management plan for mitigating dusts was implemented.

New Industrial Sources

There was 1 new industrial source relating to air quality identified within the reporting year 2025, the details are given in Table C.1.

Table C.1  – New Industrial Sources

| Permit No. | Company Name | Address | Guidance Note | Description |
| --- | --- | --- | --- | --- |
| NT174 | EG On The Move Ltd, Waterside Head Office, Haslingden Road, Blackburn, BB1 2FA, Company Number 14960308 | Cobalt PFS, Service Station, The Silverlink North, Cobalt Business Park, Newcastle, Tyne and Wear, NE27 0BY | PG 1/14 | Petrol Service Stations Part I and II |

The environmental permit was issued on the 6 August 2025. The process is vehicle vapour recovery process.  The part B permit covers the storage and handling of petrol, as well as the control of petrol vapour emissions during fuel deliveries and vehicle refuelling. These activities can release volatile organic compounds (VOCs), which contribute to air pollution and the formation of ozone. The permit applies to both petrol unloading from tankers into underground storage tanks (Stage I vapour recovery) and the refuelling of vehicles at pumps (Stage II vapour recovery), as the site’s fuel throughput exceeds the regulatory thresholds requiring a permit.

## Additional Air Quality Works Undertaken by North Tyneside Council During 2025

North Tyneside Council has not completed any additional works within the reporting year of 2025.

## QA/QC of Diffusion Tube Monitoring

North Tyneside Council uses the Gradko Laboratory for the supply and analysis of the NO2 diffusion tubes. The tubes were prepared using the 20% TEA in water preparation method. Gradko are UKAS accredited and participate in analysis scheme AIR-PT.  Monitoring is shown per calendar year.  The diffusion tube monitoring is carried out in adherence with the 2025 Diffusion Tube Monitoring Calendar.

### Diffusion Tube Annualisation

Annualisation is required for any site with data capture less than 75% but greater than 25%.  Annualisation was required for one non-automatic monitoring site, CH1 as data capture during 2025 was below 75%. Table C.2. details the calculation method undertaken.

Table C.2  – Annualisation Summary (concentrations presented in µg/m3)

| Site ID | Annualisation Factor 1 Newcastle Central | Annualisation Factor 2 Middlesbrough | Average Annualisation Factor | Annualisation Factor Site 3 | Annualisation Factor Site 4 | Raw Data Annual Mean | Annualised Annual Mean |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CH1 | 0.9711 | 0.9578 | 0.9644 | - | - | 29.1 | 28.1 |

### Diffusion Tube Bias Adjustment Factors

The diffusion tube data presented within the 2025 ASR have been corrected for bias using an adjustment factor. Bias represents the overall tendency of the diffusion tubes to under or over-read relative to the reference chemiluminescence analyser. LAQM.TG22 provides guidance with regard to the application of a bias adjustment factor to correct diffusion tube monitoring. Triplicate co-location studies can be used to determine a local bias factor based on the comparison of diffusion tube results with data taken from NOx/NO2 continuous analysers. Alternatively, the national database of diffusion tube co-location surveys provides bias factors for the relevant laboratory and preparation method.

North Tyneside Council have applied a national bias adjustment factor of 0.89 to the 2025 monitoring data, based on 28 studies as derived from the national bias adjustment factor spreadsheet 03/25 as presented in Table C.2.  A screenshot of the spreadsheet is provided in Figure C.1.

Diffusion tubes for North Tyneside Council are supplied and analysed by Gradko International Ltd who are UKAS accredited testing laboratory (No. 2187) to ISO:17025:2017. The tubes were prepared using the 20% TEA in water preparation method.

Gradko participates in the AIR PT scheme which is an internationally recognised, UKAS- accredited proficiency testing programme. Gradko has participated in quality control scheme for rounds AIR PT 68, 69, 71, 72 and 74 which covers the monitoring period for 2025. Their performance was considered satisfactory for rounds 71, 72 and 74 achieving 100% during these three rounds while less accurate in rounds 68 and 69 as shown in Table C.3.

Table C.3 	Laboratory Summary Performance for AIR NO2 PT Rounds AR068, 69, 71, 72 and 74

|  | AIR PT AR068 | AIR PT AR069 | AIR PT AR071 | AIR PT AR072 | AIR PT AR074 |
| --- | --- | --- | --- | --- | --- |
|  | January – February 2025 | April – June 2025 | July – September 2025 | 2025 October – December 2025 | January – February 2026 |
| Gradko International | 50% | 75% | 100% | 100% | 100% |

A summary of bias adjustment factors used by North Tyneside Council over the past five years is presented in Table C.4.

Figure C.1. National Bias Adjustment

Table C.4 – Bias Adjustment Factor

| Monitoring Year | Local or National | If National, Version of National Spreadsheet | Adjustment Factor |
| --- | --- | --- | --- |
| 2025 | National | 03/26 | 0.89 |
| 2024 | National | 03/25 | 0.84 |
| 2023 | National | 03/24 | 0.81 |
| 2022 | National | 03/23 | 0.83 |
| 2021 | National | 03/22 | 0.84 |

### NO2 Fall-off with Distance from the Road

Wherever possible, monitoring locations are representative of exposure. However, where this is not possible, the NO2 concentration at the nearest location relevant for exposure has been estimated using the Diffusion Tube Data Processing Tool/NO2 fall-off with distance calculator available on the LAQM Support website.

Distance correction would be considered at any monitoring site where the annual mean concentration is greater than 36µg/m3 and the monitoring site is not located at a point of relevant exposure.

No diffusion tube NO2 monitoring locations within North Tyneside required distance correction during 2025.

## QA/QC of Automatic Monitoring

North Tyneside utilities data from an automatic monitoring site operated and owned by the Urban Observatory at Newcastle University. The Urban Observatory carries out routine manual calibrations fortnightly. The supplier, Air Monitors, carries out maintenance and independent calibration on the equipment every six months. North Tyneside Council uses Ricardo to ratify the data. The Air Quality England webpage presents automatic monitoring results for North Tyneside, with automatic monitoring results also available through the UK-Air website.

### PM10 and PM2.5 Monitoring Adjustment

The type of PM10/PM2.5 monitor(s) utilised within North Tyneside do not require the application of a correction factor.

### Automatic Monitoring Annualisation

Annualisation is required for any site with data capture less than 75% but greater than 25%. All automatic monitoring locations within North Tyneside recorded data capture of greater than 75% therefore it was not required to annualise any monitoring data. In addition, any sites with a data capture below 25% do not require annualisation.

### NO2 Fall-off with Distance from the Road

Wherever possible, monitoring locations are representative of exposure. However, where this is not possible, the NO2 concentration at the nearest location relevant for exposure has been estimated using the NO2 fall-off with distance calculator available on the LAQM Support website.

No automatic NO2 monitoring locations within North Tyneside required distance correction during 2025.

## Summary of Low Cost Air Quality Sensor Monitoring

Table C.5– Details of Low-Cost Sensor Sites

| Site ID | Site Name | Site Type | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Pollutants Monitored | Sensor Type | In AQMA? | Which AQMA? (1) | Distance to Relevant Exposure (m) (1,2) | Distance to kerb of nearest road (m) (1) | Inlet Height (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HB1 | Holystone Bypass 1 | Roadside | 430489 | 570214 | NO2, PM10, PM2.5 | Zephyr | No | N/A | 6m | 1 | 6m |
| HB2 | Holystone Bypass 2 | Roadside | 430287 | 569864 | NO2, PM10, PM2.5 | Zephyr | No | N/A | 8m | 1 | 6m |
| ML | Mariners Lane | Roadside | 436391 | 569332 | NO2, PM10, PM2.5 | Zephyr | No | N/A | 6m | 1 | 6m |
| RS | Richardson Street | Roadside | 430361 | 566704 | NO2, PM10, PM2.5 | Zephyr | No | N/A | 6m | 1 | 6m |
| WL | Whitley Lodge | Roadside | 434580 | 573425 | NO2, PM10, PM2.5 | Zephyr | No | N/A | 8m | 1 | 6m |

Table C.6 – Annual and  Monthly Indicative Air Quality Sensor Monitoring Results - NO2 µg/m3

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HB1 | 430489 | 570214 | 17 | 13 | 12 | 10 | 9 | 7 | 6 | 7 | 8 | 10 | 13 | 14 | 10.5 |
| HB2 | 430287 | 569864 | 12 | 12 | 12 | 12 | 7 | 6 | 5 | 5 | 7 | 8 | 11 | 12 | 8.25 |
| ML1 | 436391 | 569332 | 30 | 19 | 19 | 16 | 14 | 14 | 10 | 12 | 15 | 15 | 19 | 20 | 16.9 |
| RS | 430361 | 566704 | 29 | 22 | 23 | 18 | 14 | 13 | 11 | 12 | 16 | 17 | 17 | 22 | 17.8 |
| WL | 434580 | 573425 | 7 | 18 | 21 | 18 | X | X | X | X | X | X | X | x |  |

Table C.7 – Annual and Monthly Indicative Air Quality Sensor Monitoring Results – PM10 µg/m3

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HB1 | 430489 | 570214 | 12 | 16 | 17 | 11 | 8 | 8 | 7 | 7 | 6 | 8 | 9 | 9 | 9 |
| HB2 | 430287 | 569864 | 8 | 14 | 16 | 16 | 8 | 7 | 7 | 7 | 5 | 7 | 9 | 8 | 9.3 |
| ML1 | 436391 | 569332 | 8 | 13 | 14 | 10 | 6 | 6 | 7 | 6 | 5 | 7 | 10 | 9 | 8.4 |
| RS | 430361 | 566704 | 14 | 16 | 17 | 11 | 8 | 7 | 7 | 7 | 5 | 8 | 8 | 9 | 9.7 |
| WL | 434580 | 573425 | 3 | 12 | 14 | 9 | X | X | X | X | X | X | X | X |  |

Table C.8 – Annual and Monthly Indicative Air Quality Sensor Monitoring Results – PM2.5 µg/m3

| Site ID | X OS Grid Ref (Easting) | Y OS Grid Ref (Northing) | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HB1 | 430489 | 570214 | 7 | 10 | 10 | 8 | 6 | 5 | 5 | 5 | 3 | 5 | 5 | 5 | 6.2 |
| HB2 | 430287 | 569864 | 5 | 9 | 10 | 10 | 5 | 5 | 5 | 5 | 3 | 5 | 5 | 5 | 6 |
| ML1 | 436391 | 569332 | 12 | 9 | 9 | 7 | 4 | 4 | 4 | 4 | 3 | 4 | 6 | 5 | 5.9 |
| RS | 430361 | 566704 | 9 | 10 | 11 | 7 | 5 | 5 | 5 | 5 | 3 | 5 | 5 | 6 | 6.3 |
| WL | 434580 | 573425 | 3 | 8 | 9 | 5 | X | X | X | X | X | X | X | X |  |

# Appendix D: Map(s) of Monitoring Locations and AQMAs

Figure D.1 – Map of Automatic and Non-Automatic Monitoring Site

Figure D.2 – Map of Indicative Air Samplers Monitoring Locations

# Appendix E: Summary of Air Quality Objectives in England

Table E.1 – Air Quality Objectives in England

| Pollutant | Air Quality Objective: Concentration | Air Quality Objective: Measured as |
| --- | --- | --- |
| Nitrogen Dioxide (NO2) | 200µg/m3 not to be exceeded more than 18 times a year | 1-hour mean |
| Nitrogen Dioxide (NO2) | 40µg/m3 | Annual mean |
| Particulate Matter (PM10) | 50µg/m3, not to be exceeded more than 35 times a year | 24-hour mean |
| Particulate Matter (PM10) | 40µg/m3 | Annual mean |
| Sulphur Dioxide (SO2) | 350µg/m3, not to be exceeded more than 24 times a year | 1-hour mean |
| Sulphur Dioxide (SO2) | 125µg/m3, not to be exceeded more than 3 times a year | 24-hour mean |
| Sulphur Dioxide (SO2) | 266µg/m3, not to be exceeded more than 35 times a year | 15-minute mean |

# Glossary of Terms

| Abbreviation | Description |
| --- | --- |
| AQAP | Air Quality Action Plan - A detailed description of measures, outcomes, achievement dates and implementation methods, showing how the local authority intends to achieve air quality limit values’ |
| AQMA | Air Quality Management Area – An area where air pollutant concentrations exceed / are likely to exceed the relevant air quality objectives. AQMAs are declared for specific pollutants and objectives |
| ASR | Annual Status Report |
| Defra | Department for Environment, Food and Rural Affairs |
| DMRB | Design Manual for Roads and Bridges – Air quality screening tool produced by National Highways |
| LAQM | Local Air Quality Management |
| NO2 | Nitrogen Dioxide |
| NOx | Nitrogen Oxides |
| PM10 | Airborne particulate matter with an aerodynamic diameter of 10µm or less |
| PM2.5 | Airborne particulate matter with an aerodynamic diameter of 2.5µm or less |
| QA/QC | Quality Assurance and Quality Control |
| SO2 | Sulphur Dioxide |

# References

- Local Air Quality Management Technical Guidance LAQM.TG22. August 2022. Published by Defra in partnership with the Scottish Government, Welsh Assembly Government and Department of the Environment Northern Ireland.

- Local Air Quality Management Policy Guidance LAQM.PG22. August 2022. Published by Defra in partnership with the Scottish Government, Welsh Assembly Government and Department of the Environment Northern Ireland.

- Chemical hazards and poisons report: Issue 28. June 2022. Published by UK Health Security Agency

- Air Quality Strategy – Framework for Local Authority Delivery. August 2023. Published by Defra.

Defra. Air Quality Strategy – Framework for Local Authority Delivery, August 2023

The units are in microgrammes of pollutant per cubic metre of air (µg/m3).